# Speeds for Normal Operation

VX	65 KIAS	= Best Angle of Climb Speed

VY	79 KIAS	= Best Rate of Climb Speed

Vglide	75 KIAS	= Best Glide (lift/drag), flaps up

Vglide0	65 KIAS	= Best Glide (lift/drag), flaps down

VS	54 KIAS	= Stall Speed, normal configuration

VS0	45 KIAS	= Stall Speed, landing configuration

VFE	115/90 KIAS	= Maximum Flap Extension Speed

VA	102 KIAS	= Maneuvering Speed (at 2500 lbs)

VNO	138 KIAS	= Maximum Structural Cruising Speed

VNE	167 KIAS	= Never Exceed Speed

65-75 KIAS	= Normal Climb Out

57 KIAS	= Short Field T/O, Flaps 15°

70-80 KIAS	= Normal Landing Approach, Flaps Up

60-70 KIAS	= Normal Landing Approach, Flaps 30°

61 KIAS	= Short Field Approach, Flaps 30°

Engine Failure During Takeoff

- Throttle	IDLE

- Brakes	APPLY

- Wing Flaps	UP

- Mixture	IDLE CUT-OFF

- Ignition Switch	OFF

- Master Switch	OFF

Engine Failure After Takeoff

- Airspeed	 70 KIAS

- Mixture	IDLE CUT-OFF

- Fuel Selector Valve	OFF

- Ignition Switch	OFF

- Wing Flaps	AS REQUIRED

- Master Switch	OFF

Engine Failure During Flight

- Airspeed	75 KIAS

- Carburetor Heat	ON

- Fuel Selector Valve	BOTH

- Fuel Shutoff Value	ON

- Mixture	RICH

- Aux Fuel Pump	ON 3-5 sec (throttle 1/2”), OFF

- Ignition Switch	BOTH or START

Emergency Landing

- Airspeed	75 KIAS

- Mixture	IDLE CUT-OFF

- Fuel Selector Valve	OFF

- Ignition Switch	OFF

- Wing Flaps	30 or AS REQUIRED

- Master Switch	OFF

- Doors	UNLATCH

- Touchdown	SLIGHTLY TAIL LOW

- Brakes	APPLY HEAVILY

Precautionary Landing

- Airspeed	65 KIAS

- Wing Flaps	15

- Select field	SUITABLE

- Avionics Power Switch	OFF

- Electrical Switches	OFF

- Wing Flaps (on final approach)	30

- Airspeed	65 KIAS

- Master Switch	OFF

- Doors	UNLATCH

- Touchdown	SLIGHTLY TAIL LOW

- Ignition Switch	OFF

- Brakes	APPLY HEAVILY

## Cabin

- Aircraft Documents & POH	 AVAILABLE

- Control Wheel Lock	REMOVE

- Ignition Switch	OFF

- Radio Master Switch	OFF

- Master Switch	ON

- Fuel Quanity Indicators	CHECK QUANTITY

- Wing Flaps	DOWN

- Master Switch	OFF

## Empennage

- Tail Tie-Down	DISCONNECT

- Control Surfaces	CHECK FREE

- Static Ports 	CHECK

## Right Wing

- Aileron	CHECK FREE

- Fuel Tank Vent	CHECK

- Wing Tie-Down	DISCONNECT

- Main Wheel Tire	CHECK

- Fuel Tank Sump	SAMPLE

- Fuel Quantity	CHECK

- Fuel Filler Cap	SECURE

## Nose

- Engine Oil Level	CHECK

- Fuel Strainer	DRAIN

- Propeller and Spinner	CHECK

- Landing & Taxi Lights	CHECK

- Carburetor Air Filter	CHECK

- Nose Wheel Strut and Tire	CHECK

- Static Ports	CHECK

- Fuel Sump	SAMPLE

## Left Wing

- Fuel Tank Sump	SAMPLE

- Fuel Quantity	CHECK

- Fuel Filler Cap	SECURE

- Main Wheel Tire	CHECK

- Stall Warning	TEST

- Wing Tie-Down	DISCONNECT

- Pitot Tube Cover	REMOVE

- Fuel Tank Vent	CHECK

- Aileron	CHECK FREE

## Before Starting Engine

### Preflight Inspection	COMPLETE

### Seats, Belts & Harnesses	ADJUST & LOCK

### Fuel Selector Valve	BOTH

### Fuel Shutoff Valve	ON

### Avionics Power Switch	OFF

### Electrical Equipment	OFF

### Circuit Breakers	CHECK IN

### Brakes	TEST & SET

### Cowl Flaps	OPEN

### Passengers	BRIEF

## Starting Engine

- Hobbs Time 	 RECORD

- Mixture	RICH

- Propeller 	HIGH RPM

- Carburetor Heat	COLD

- Master Switch	ON

- Beacon	ON

- Prime	AS REQUIRED

- Throttle	OPEN 1/2”

- Propeller Area	CLEAR

- Ignition Switch	START

- Throttle	1000 RPM

- Mixture	LEAN

- Oil Pressure	CHECK

- Panel & Instrument Lights	ADJUST

- Taxi & Navigation Lights	AS REQUIRED

- Avionics Power Switch	ON

- Radios	ON

- Transponder	STANDBY

## Before Taxi

- Wing Flaps	UP

- Parking Brake	RELEASE

- Brakes	TEST

- Area	CLEAR

## Before Takeoff

- Parking Brake	SET

- Doors & Windows	CLOSED & LOCKED

- Flight Controls	FREE & CORRECT

- Flight Instruments	SET

- VOR NAV Radios	COMPARE & LOG

- Autopilot	TEST

- Fuel Shutoff Valve	ON

- Fuel Selector Valve	BOTH

- Mixture	(below 3000’) RICH

- Auxiliary Fuel Pump	CHECK

- Stabilator & Rudder Trim	TAKEOFF

- Throttle	1800 RPM

- Magnetos	CHECK

- Propeller 	 CYCLE

- Carburator Heat	CHECK

- Engine Instruments	CHECK

- Ammeter	CHECK

- Suction Guage	CHECK

- Throttle	1000 RPM

- Throttle Friction	SET

- Radios	SET

- Autopilot	ON/DISENGAGED

- Strobe Lights	AS REQUIRED

- Transponder	ALT

## Normal Takeoff

- Wing Flaps	0-10

- Carburetor Heat	COLD

- Power	FULL & 2700 RPM

- Rotate	50 KIAS

- Climb Speed	65-75 KIAS

- Wing Flaps	RETRACT

## Short Field Takeoff

- Wing Flaps	15

- Carburetor Heat	COLD

- Brakes	APPLY

- Power	FULL & 2700 RPM

- Mixture	(below 3000’) RICH

- Brakes	RELEASE

- Rotate	50 KIAS

- Climb Speed	(until obstacles cleared) 57 KIAS

- Wing Flaps	(slowly) RETRACT

## Enroute Climb

- Airspeed	75-85 KIAS

- Power	24” or FULL & 2500-2700 RPM

- Mixture	(below 3000’) RICH

- Cowl Flaps	AS REQUIRED

## Cruise

- Power	15-24” & 2100-2700 RPM (75% or less)

- Stabilator & Rudder Trim	ADJUST

- Mixture	LEAN

- Cowl Flaps	CLOSED

## Descent

- Power	AS DESIRED

- Mixture	RICH or LEAN

- Carburetor Heat	AS REQUIRED

- Cowl Flaps	CLOSED

## Before Landing

- Seats, Belts & Harnesses	SECURE

- Fuel Selector Valve	BOTH

- Mixture	RICH

- Carburetor Heat	ON

- Propeller	HIGH RPM

- Airspeed	(flaps up) 70-80 KIAS

- Flaps	AS DESIRED

- Airspeed	(flaps down) 60-70 KIAS

- Stabilator & Rudder Trim	ADJUST

## Balked Landing

- Power	FULL & 2700 RPM

- Carburetor Heat	COLD

- Wing Flaps	20

- Climb Speed	65 KIAS

- Wing Flaps	(slowly) RETRACT

- Cowl Flaps	OPEN

## After Landing

- Wing Flaps	UP

- Carburetor Heat	COLD

- Cowl Flaps	OPEN

- Transponder	STANDBY

## Shutdown

- Parking Brake	SET

- Avionics Power Switch	OFF

- Electrical Equipment	OFF

- Mixture	IDLE CUT-OFF

- Ignition Switch	OFF

- Master Switch	OFF

- Control Lock	INSTALL

- Fuel Selector Valve	RIGHT

# Specifications

2500 lbs	= Maximum Gross Weight

1598 lbs	= Basic Empty Weight

608 lbs	= Payload With Full Fuel

10.0 GPH	= Fuel Flow @ 75% Power

50 gal	= Total Fuel Capacity

49 gal	= Total Useable Fuel

100LL	= Fuel Octane Rating (Blue)

8 qts	= Oil Sump Capacity

6 qts	= Minimum Oil Quantity

180 hp	= Lycoming O-360-A1F6

12 V	= Battery

Pretty Good Planes, LLC.

4360 Headen Way

Santa Clara, CA 95054

Attn: Wilfredo Sánchez Vega

### Email:	pilots@prettygoodplanes.com

### Internet:	www.prettygoodplanes.com